Metabolic Activity Diffusion Imaging [MADI]:

I.  Metabolic, Cytometric Modeling and Simulations

Charles S. Springer, Jr.,*a-e Eric M. Baker,a† Xin Li,a,c† Brendan Moloney,a† Gregory J. Wilson,f†

Martin M. Pike,a,d  Thomas M. Barbara,a  William D. Rooney,a,d,g Jeffrey H. Makih

†These authors, listed in alphabetical order, contributed equally.

aOregon Health & Science University [OHSU] Advanced Imaging Research Center, bOHSU Knight Cancer Institute, cOHSU Brenden-Colson Center for Pancreatic Care, dOHSU Department of Biomedical Engineering, eOHSU Department of Chemical Physiology and Biochemistry, fUniversity of Washington Department of Radiology [current: Bayer Healthcare, Radiology], gOHSU Department of Behavioral Neuroscience, hUniversity of Colorado Anschutz Medical Center Department of Radiology.

*Corresponding author:

Charles S. Springer, Jr.

Advanced Imaging Research Center, L452

Oregon Health and Science University

3181 S.W. Sam Jackson Park Road

Portland, Oregon 97239

503-418-1531

springer@ohsu.edu

Supplementary Material

Supplementary Material [citations 1-85 in main text]:

S.I.  Creating Digital Cell Ensembles: The Contracted Voronoi Cell.  A standard Voronoi cell, Ci, is defined to be the volume comprising all points closer to the cell’s seed point, ci, than to any other seed point:

Ci=x∈R3  distx, ci<distx,cj for all j≠i#[S1]

where distx, y denotes the Euclidean distance between points x and y.  Therefore, an ensemble of standard Voronoi cells fills all space (86).  An elaboration is shown in Figure S1, which depicts a 2D analog of a cell ensemble [shaded], and four concentric cell domains [Ω, square boundaries], intracellular volume fraction, vi = 1, and mean cell volume, V = 1/ [the cell number density reciprocal] for infinitely thin cell boundaries.  Of course, the actual tessellation is three dimensional.

However, our diffusion simulations require [interstitial] space between the cells [Fig. 2 in text].  In this section, we describe how we “create” this space by contracting standard Voronoi cells.  More elaborations of Fig. S1 are given below.

To produce a contracted Voronoi cell, we divide a standard cell into an inner shrunken cell, Ci(cell), and its’ enveloping annulus, Ciann.  This division is accomplished by specifying an annulus width, αi, by which the bounding planes of the standard Voronoi cell are translated towards the cell’s seed point.  The contracted cell is the volume comprised of all points remaining internal to the translated planes:

Cicell=x∈R3  distx, ci<distx,cj+2αici-cjdistci,cj for all j≠i#[S2]

The portion of the original standard Voronoi cell not within the shrunken cell is taken to be the annulus:

Ciann=Ci∖Cicell#[S3]

[Ciann represents all elements in Ci, but not also in Cicell].  In an ensemble of cells, the union of all annuli is taken in our simulations to be the fully connected ensemble interstitial [extracellular] space.

While each cell’s annulus width could be individually assigned, we implement contraction through specifying a single, global target annulus width, α*.  The annulus width is set to α* for all those cells for which that does not result in moving any of the bounding planes past the cell’s seed point.  To prevent the latter

Figure S1.  A 2D representation of standard Poisson-Voronoi tessellation, with punctile cell seeds.  In this, the cells fill all space [intracellular volume fraction, vi = 1; and mean cell volume, V = 1/, the cell number density reciprocal].  Our random walk simulations, however, are conducted in 3D ensembles of contracted Voronoi cells [Fig. S2].  The shaded cells here represent an analog of a 3D ensemble.  The four squares represent analogs of 3D domains discussed in detail in Section S.III.  The dimension W is typically near 500 μm.  Thus, the cubical domain Ωsim typically has a volume (W3) near 125 μL.  Such tissue volumes typically contain many thousands of cells [text].  Thus, it is important to note that [for clarity] the cells are not drawn to scale here.

scenario, we impose an upper limit, αi(lim), on the annulus width of an individual cell: i must be the lesser of

αi=minα*, αilim#[S4]

* and i(limit).  The value of this limit is a specified fraction, κ∈[0,1), of the width that would result in a bounding plane moving past the cell’s seed point:

αilim=12κ min distci,cj  for all i≠j#[S5]

An example of a contracted Voronoi cell, C5(cell), is illustrated in Figure S2.  The grey dashed lines mark the boundaries of the original cells with adjacent standard Voronoi cells.  The central pink area is C5(cell), and the inner red dotted outline is the maximum extent to which C5(cell) could be contracted based on the annulus limit,  α5lim, resulting from seed c4 being the nearest neighboring seed to c5.  The sum of 5 and 4 is the interstitial “gap” between contracted cells 5 and 4.  If j of a bordering cell Cj is  αjlim, the gap at the j face of a cell can be different from the gaps at its other faces [not shown in Fig. S2].

S.II.  Properties of the Stochastic Geometry: The Use of Single-Celled Ensembles.  The randomness in contracted Voronoi cell geometries is driven by the underlying process governing the spatial dispersion of seed points.  We have chosen the Poisson point process to effect this dispersion (86).  Thus, all possible points in space have an equal, independent probability of being a seed point.  Therefore, for a 3D domain

Ω∈R3 with volume VΩ, the probability of finding n seed points within Ω is given by a Poisson distribution:

Pn=ρ(VΩ)nn!exp-ρ(VΩ)    for n=0,1,…#S6

The coupling of the Poisson point process with standard Voronoi partitioning is known as Poisson-Voronoi tessellation, and is well-studied (86).  To explore and characterize the properties of interest for our Poisson-contracted-Voronoi tessellation, 5x106 independent, single-celled ensembles [i.e., ρ=1] with κ=0.9 were generated.  Picture the 3D version of only Fig. A2 C5, with its annulus.  The procedure described in (86) was followed to generate the initial uncontracted Voronoi cell of each single-celled ensemble.  For the resulting single contracted cell of each ensemble, the surface area, A, and volume, V, were computed using the convex hull routines of the SciPy package (25,87), for each of 26 linearly-spaced α* values.  Results

Figure S2.  The scheme for contracting Voronoi cell C5.  The cell’s annular thickness relative to neighboring cell C4 is denoted 5.  The process is described in detail in Sec. S.I.  [5 + 4] is the interstitial “gap” between cells C5 and C4.

derived from the five million cells are seen in Figures S3 to S7.

Figure S3a shows the dependence of the computed intracellular volume fraction, vi (26 orange points), on the cell density-normalized target annulus width, ρ13α*, which is dimensionless.  This relationship can be approximated with the empirical cubic polynomial expression in Equation [S7a] (blue curve).

vi=EVCjcell  α*EVCjcell  0≈- 0.570444ρ13α*3+10.6047ρ13α*2-5.81457ρ13α*+1#S7a

The root-mean-squared error of the fitting is 2.13×10-5 and the maximum absolute error is 5.74×10-5.  As expected, the vi parameter clearly decreases with increasing annulus width.

For multi-celled ensembles, vi = V, and Eq. [S7a] can be re-written as Equation [S7b].  For fixed ,

V ≈ - 0.570444α*3+10.6047ρ-1/3α*2-5.8147ρ-1/32α*+ ρ-1/33    		  [S7b]

V varies as a cubic expression in *.  Figure S3b shows the result for extension to pertinent multi-celled ensembles.  It presents a family of such plots for different fixed  values.  For these, the ordinate V and abscissa * now have dimensions.  For example, with  fixed at a typical tissue value of 5 x 105 cells/μL (red), it is very clear V decreases from 2.0 to 0.7 pL as * increases from 0 [vi = 1] to 1.95 μm.

Alternatively, for fixed * in Eq. S7b, V varies as a cubic expression in the reciprocal cube root of  (which has the dimension of length).  In Figure S3c, a family of hyperbolae shows the -dependences of V for different fixed * values.  One could select the  value desired for a contracted Voronoi cell domain, and then determine the * value necessary to produce the V value desired.  For this, Eq. [S7b] could be used for interpolation between the * curves shown.  Though the approach in Fig. S3b was originally conceived, Fig. S3c shows another way to understand the creation of domains with desired  and V values.  It could allow a continuous ,V parameter space.

Further illustrations of results from the individual studies of 5x106 different single-celled ensembles are assembled in Figure S4.  Panel a shows the probabilities of finding density-normalized cell volume, V(C(cell)), values for different specified vi values.  For multi-cell ensembles, vi will also be a random variable with expectation Evi | α*=ρEVCjcell  α*.  Extrapolating to the specific multi-cell ensemble

Figure S3.  (a) The intracellular volume fraction, vi, as a function of the cell density ()-normalized global target cell annulus thickness (*).  Each of the 26 orange points is the vi value averaged from analyses of 5x106 single contracted Voronoi cells, with their annuli [imagine Fig. S2 cell C5 in 3D.]  Thus, this is the general case.  The blue curve is from the fitting of cubic Eq. [S7a], with parameters given in Sec. S.II.  (b)  A family of specific cases [extended from (a)] for multicellular ensembles with three different values of fixed , indicated by the colored iso- contours.  V decreases as * increases from 0 [vi = 1].  (c) A family of specific cases for multicellular ensembles with three different values of fixed *, indicated by the colored iso-* contours.  Thus, V is plotted vs. : a section of V parameter space.  The colored curves are calculated from cubic Eq. [S7b].  V decreases with increasing .  Practical biological tissue values range from: 0.02 to 10 pL for V and 50,000 to 5,000,000 cells/μL for .  For further insight, hyperbolic iso-vi contours are also plotted [darker gray].  No V combinations are allowed above V = vi = 1.0

Figure S4. a) The probability density functions [P] of cell density-normalized cell volumes for 5x106 single [i.e.,  = 1] contracted Poisson-Voronoi cells.  The individual curves are labeled with the desired global intracellular volume fraction (vi) values for the distributions.  For example, there is a distribution of V’s, each having vi = 1.  This is the general case.  b) The specific case for  = 500,000 cells/μL [i.e., vs. V(C(cell))].

Figure S5.  (a) The -normalized cell area, A, dependence on the -normalized target annulus, *.  The orange points are averaged from 5x106 single contracted Voronoi cells, with their annuli [imagine Fig. S2 cell C5 in 3D.].  The blue curve is from fitting with parameters given in Sec. S.II.    This is the general case.  A decreases with increasing *.  (b) The –dependences of the A product for specific multicellular ensembles at three different * values.  The A product is ae [the cell surface area/ensemble volume]; a measure of the ensemble membrane “concentration.”  It is analogous to vi.  We see ae increases with increasing .

Figure S6. (a) The vi-dependences of differently-averaged -normalized cellular claustrophobia ratio (CR = A/V) values for 5x106 contracted Voronoi cells.  These are important for determining kio, and are discussed in detail in Section S.IV.  This is the general case.  (b) The V-dependences for the specific case of  = 500,000 cells/μL.  In addition, the purple dashed curve shows the V-dependence of 1/V, right ordinate.

Figure S7.  (a) The vi-dependences of the ratios of differently averaged cellular claustrophobia ratio (A/V) values for 5x106 contracted Voronoi cells.  These are important for determining kio, and are discussed in detail in Sec. S.IV.  This is the general case.  (b) The V-dependences for the specific case of   = 500,000  cells/μL.]  Note: the ratios are all close to unity.

case of  = 5x105 cells/μL, Fig. S4b shows the probabilities for different desired V values.  These resemble  distributions: V cannot be < 0.

Figure S5 is analogous to Fig. S3, except A replaces V in all instances.  Thus, the ordinate of Fig. S5a is 2/3A, and the abscissa is 1/3*.  For Fig. S5b, the ordinate is ae, defined as A.  This is the claustrophobia ratio, A(Cj(cell))/V(Cj(cell)), in single-celled ensemble Cj(cell) with * = 0.  The abscissa is , and there is a family of curves for different * values [analogous to Fig. S3c].  ae can be thought of as representing the ensemble surface “concentration.”  It increases with ρ.  Along with the permeation permeability, pp, this is what our simulation’s success suggests is the major impediment to water diffusion in tissue.

S.III.  Creation of Contracted Voronoi Cell Ensembles for Desired  and V values.  The simulation domain, Ωsim, is a cube with edges of length W (Fig. S1, for standard Voronoi).  If a molecule exits Ωsim at any point during a walk, the simulation immediately ceases, reporting an error.  All molecular walks originate in the inner cubical [source] domain, Ωsrc, with edges of length 0.4W (Fig. S1, for standard Voronoi).

The specific value of W we use is determined with the Equation [S8] heuristic

W=max10LRW , min20LRW ,38×105ρ #S8

where:

LRW=2D0tRW#S9

is the diffusion trajectory length over the temporal extent of the random walk.  [Note: this is not the same as the net diffusion displacement, dms = 6D0tRW (Fig. 3).]  A typical W value is 438 μm.  Use of this heuristic has been found to sufficiently eliminate cases of molecules leaving Ωsim, while also satisfactorily reducing the computation time by allowing Ωsim domains with sufficiently curtailed numbers of cells.  A (438 μm)3 tissue domain has a volume, VΩsim= 84 nL [0.084 μL], and contains many thousands of cells [see text and Fig. 1].  Thus, for clarity, the cells in Fig. S1 are not drawn to scale.

A single cell ensemble created to tessellate Ωsim is a random realization of the contracting Poisson-Voronoi process.  It should be emphasized that any particular multicell ensemble generated for a desired ρ and V does not necessarily have an amount of cell seeds equal to the expected [desired] number, En|ρ,Ωsim=ρVΩsim, or the desired mean cell volume V.  Ensemble cells protruding from sim faces, many of their seeds, and major portions of their volumes, are therefore not counted in sim.  Stated in another manner, ρ and V values derived from a collection of cell ensemble realizations will be equal to those specified in the limit of an infinite collection.  Equally, a cell ensemble will have the specified ρ and V values in the limit of [VΩsim]→∞.  However, because the Ωsim domain contains many thousands of cells, the percentage  and V uncertainties are very small.

The specified ρ and V values are the desired quantities for a Poisson-contracted Voronoi ensemble.  But, we can find the more directly-implementable parameter α* that corresponds to these ρ and V values by solving Eqs. [S7] with vi=ρV.  There will be at most three unique α* solutions but only one in the range of α* values desired.  This is due to Eq. [S7a] being a bijection (as can be seen in Fig. S3a) over the range in question.  Every realization with the same specified ρ and V will use the same α*, but the application of α* (Eq. [S4]) does not happen until after a random standard Voronoi realization for a desired set of cell seeds has been generated (Fig. S1).

Valid tessellation of Ωsim with contracted Voronoi cells necessitates that a larger, encompassing domain, Ωpop⊃Ωsim, be populated by a set of seed points, Φ, satisfying the Poisson point process for desired ρ.  Cells with seed points outside Ωsim can have portions that protrude into Ωsim.  Accounting for this possibility requires that, for every point on the  Ωsim boundary (∂Ωsim) there is a point in the collection of seed points within Ωpop that is closer to it than to any point outside of Ωpop:

Ωpop⊇x∈R3  Φ ∃ x'∈∂Ωsimabdistx, x'<distci,x' ∀ ci∈Φ∩Ωsim#S10

The nature of contracted Voronoi cells – with annuli - imposes an additional requirement.  The limit on the annulus width depends on the distance between the cell’s seed point and the nearest-neighboring seed point.  Ensuring that all cells within or protruding into Ωsim have an unbiased annulus width leads to the requirement that, for these cells, no point outside Ωpop can be closer to the cell’s seed point than the nearest neighboring seed point within Ωpop:

Ωpop⊇x∈R3  ∃ ci∈Φabdistx, ci<distci,cj ∀ cj∈Φ∖{ci}#S11

where Φ is the subset of seeds for those cells inside or overlapping Ωsim:

Φ=ci∈Φ  Ci∩Ωsim≠∅#S12

Figure S1 illustrates these concepts in 2D with standard Poisson-Voronoi tessellation.  The red square represents Ωsrc, the black square Ωsim.  The shaded cells are all those within or protruding from the Ωsim boundary.  The blue dots and “x”s are cell seeds: the former represent the seed set influencing partitioning within Ωsim.  These comprise seeds of either the shaded cells or the shaded cells’ nearest neighbors.

The orange and green squares are formulated under the constraint that Ωpop be a square concentric with Ωsim.  The region enclosed by the orange square is labeled Ω1c [i.e., pop = 1c], and is the minimal Ωpop necessary to satisfy Eq. [S10] when only the seed points within Ωsim are known.  The orange dashed circle marks the case that defines the extent of the orange square.  The center and radius of the circle are, respectively, the point on ∂Ωsim farthest from a seed within Ωsim and the distance to that seed.

The region enclosed by the green square labeled Ω2c [i.e., pop = 2c] is the minimal Ωpop necessary to satisfy Eq. [S11] when only the seed points within Ω1c are known.  Ω2c must enclose all circular regions within which a point would be closer to a cell seed, of those seeds for cells that are within or overlap Ωsim, than to the nearest neighboring seed in Ω1c.  Of these circular areas, the green dashed circle is the one that enlarges Ω2c to its maximum extent.

S.IV.  Specifying kio for a Random Walk in a Given Ensemble.  As detailed in the text, Eq. [5] is crucial for the MADI approach.  It shows that to specify the average kio for a random walk, kio, in a given ensemble, one needs the permeation probability, pp, and the ensemble-averaged claustrophobia ratio, AV.  As with the desired ρ and V values for a realized cell ensemble, AV is taken as that of the governing contracted Poisson-Voronoi generating process.  Thus, it is not necessarily the A/V in Ωsim, where the walking will occur.  Thus, much like the determination of vi for α* above, the value for AV is interpolated from those determined for the five million single, contracted Voronoi cell ensembles.  For the 26 * values, we compute AVα*= 15,000,000i=15,000,000Aiα*Viα*.  From the 26 A/V values we interpolate that for an ensemble with our desired vi value.

Figure S6a shows the vi-dependence of the cell density-weighted AV so determined [blue curve] on the intracellular volume fraction, vi.  The plot of the ratio of the averages, A/V, is also shown [green curve].  It is very important to note that AV≠A/V.  A/V is appropriate for determination of kio for random walks with a given pp in a given ensemble.  The harmonic mean, V/A-1, would be appropriate for the reciprocal of the mean intracellular water molecule lifetime i-1.  The harmonic mean plot is also shown [red curve] in Fig. S6a.  The lifetime i was reported for many early trans-cytolemmal water exchange studies, with the caveat that kio ≠ 1/i (6).  Figure S6b shows the V-dependences of three means for the specific tissue case of  = 5 x 105 cells/μL.  Now, A/V must have units.  The claustrophobia ratio A/V is seen to fall from 0.80 to 0.50 μm-1 as V increases from 0.75 to 2.00 pL.  We previously discussed the A/V decrease with increasing V in general qualitative terms (4).]  We also show the V-1 decrease is similar.

The vi-dependences of the ratios V/A-1/A/V and (A/V)/A/V are shown as the blue and orange curves, respectively, in Figure S7a.  It is important to note the different means are not too dissimilar under biological conditions.  A/V is reduced from A/V by at most by 15%, and then only at vi values unreasonably small for most tissues.  The V-dependences of these ratios are shown in Fig. S7b for the specific tissue case of  = 5 x 105 cells/μL.

S.IV.a.  Determining a molecule’s position and compartmental environment within an ensemble.

The procedure used for simulating a random walk (Fig. 2) requires knowing within which compartment (i.e., which  Cicell or Ciann) a molecule is currently located, and in which compartment it will be at the end of the subsequent time step.  This does not depend on what lies along the path between the positions at two successive time steps.  The compartmental information is obtained by querying a kD-tree for the two nearest cell seeds, c1 and c2, and their distances, distx,c1≤dist(x,c2), to the position, x, of the molecule.  See Figure S8.  [When i is computed before each simulation, only the coordinates of seeds like those represented by the Fig. S1 blue dots are needed.  During simulation, the seeds of only the Fig. S1 shaded cells are required.]  By definition (Eq. [S1]), x∈C1.  The C1 sub-compartment in which x is located is determined by

x∈C1cell,if α'≥α1C1ann,otherwise #S13

where  is the perpendicular distance to the boundary plane separating the interstitial annuli of the two cells

α'=distx,c22-distx,c122 distc1,c2#S14

with seeds nearest the molecule: A diagram of the geometric foundation of Eq. [S13] and Eq. [S14] is presented in Fig. S8.

Figure S8.  A geometric perspective of how the spatial position of a walking molecule at position x is indexed relative to the seeds of adjacent contracted Voronoi cells [c1, c2].  At the instant pictured, the molecule is in the interstitial [extracellular] gap between cells C1 and C2 [in the C1 annulus].  See Sec. S.IV.b.1.

S.IV.b.  Simulation of a random walk.  Figure S9 presents the logic described below in the form of a flowchart.

S.IV.b.1.  Location in space.  A molecule walk starts at random location xk=0 within domain Ωsrc.  A potential position, xk+½, for the molecule at the next time step is drawn from a three-dimensional normal distribution having mean, xk, and component variance, 2D0ts.  As given in the text, the mean separation of xk and xk+1/2 is (12D0ts/)1/2 [the median separation is (12D0ts)1/2erf-1(1/2)].  The specific compartment of this potential position is determined (Eq. [S13]).

If xk+1/2 is inside a cell while xk is in that cell’s gaps, or if xk+1/2 is in a gap while xk is inside a bordering cell, the molecule will have to cross one membrane [m = 1] in traversing between the two positions.  If xk is inside a cell while xk+1/2 is inside an adjacent cell, the molecule will have to cross two membranes [m = 2] in traversing between the two positions.  If neither of these two conditions is true, the molecule either stays in the extracellular space or stays inside the same cell.  The number of membrane crossings for this case is zero [m = 0].

A random number between 0 and 1 is drawn to determine whether the molecule will successfully permeate the membranes required to get to xk+1/2.  If the random number is between ppm and 0, the molecule successfully crosses the membranes, and the molecule’s position set to xk+1/2.  If not successful, the molecule’s position at the next time step remains at xk.  The molecule is not allowed angular reflections off the cell boundary.

A practical example of this is the situation for water molecules within the neuronal myelin sheath, where there are many cell membranes [m is large] in close series (1).  Although there is reasonably rapid exchange of water in and out of the sheath, the probability on any one molecule completely traversing the entire sheath is very small: ppm decreases exponentially with m.

S.IV.b.2.  Location in time.  After each time step, the elapsed time is then advanced.  When the elapsed time reaches tRW,max [80 ms, here], the random walk is concluded.  If not, the sampling of the potential position begins again for the next time step.

Figure S9.  A flow chart for the simulation of a molecular random walk through an ensemble of contracted Voronoi cells.  See Sec. S.IV.b. for a discussion.

S.VI.  Interpreting the SDE Decay.

Though there are analytical [i.e., equation-based] methods for fitting experimental SDE decays, these are all empirical: the parameters cannot be assigned as actual physical properties that might be independently assayed.  Furthermore, one or two parameter expressions are insufficient.  Gaussian diffusion can be characterized with one parameter, the apparent diffusion coefficient [ADC], in the equation: ln[S(b)/S0] = e-b(ADC).  However, Gaussian behavior does not obtain for almost any biological tissue (8).  The diffusion kurtosis expressions have two parameters; Dap and Kap (8).

There have been many demonstrations that three parameter empirical expressions are sufficient to describe experimental SDE decays (23).  Perhaps the most popular is the general bi-exponential Eq. [10], re-written here as Equation [S15], where pA is the mole fraction [“population”] of molecules exhibiting

SbS0 =  pAe-bDA+ 1- pAe-bDB 					[S15]

diffusion coefficient DA, and all other molecules exhibit DB.  This expression has three variable parameters: pA, DA, and DB [pB = 1 – pA], which are taken as time-independent.  Equation [S15] implicitly assumes there is no molecular exchange between the populations [i.e., kAB = 0], which is almost never the case.  More importantly however, every realistic tissue diffusion goes through an essentially infinite number of D values during the diffusion time, tD [= tRW; Figs. 3b,3d,3e,3f].  The significant hazards of inappropriate empirical bi-exponential interpretations are quite perilous (81).  Attempts to assign physical quantities to pA, DA, and DB can lead to very serious errors (2,17): these are empirical parameters.

S.VII.  Kärger/Andrasko Model and its’ Descendants.

The Kärger/Andrasko [KA] Model [reviewed in (17)] was introduced starting in the 1960’s to add the potential for steady-state trans-cytolemmal molecular exchange by altering the empirical bi-exponential expression, Eqs. [10] and [S15] with a two-site-exchange [2SX] formalism.  We do not use KA in the MADI approach [for reasons given below].  However, since it has been employed in almost all DWI models incorporating exchange (88), and its’ concepts are heuristic, we give a brief recapitulation.  Equations [10] and [S15] are re written more explicitly [in MADI notation] in Equation [S16], where pi, Di, po, and Do are

SS0= pie-bDi+ poe-bDo= pie-q2DitD + poe-q2DotD= pie-q2DitD+ poe-q2DotD

[S16]

the apparent intra- and extracellular water mole fractions and diffusion coefficients [i.e., as affected by exchange].  These are expanded in terms of the intrinsic quantities [sans exchange] in the Bloch-McConnell-Woessner [BMW] Equations [S17-S20] (2).  The matrix bases for these integrated

q2Di= 12q2Di+ q2Do+k- 12argDio+ kio- koi2+4kiokoi12                             [S17]

q2Do= 12q2Di+ q2Do+k+ 12argDio+ kio- koi2+4kiokoi12                            S18

pi= 12+ 12argDiopo- pi+kargDio+ kio- koi2+4kiokoi12                                                    [S19]

po= 12- 12argDiopo- pi+kargDio+ kio- koi2+4kiokoi12                                                   [S20]

differential expressions, as well as the case including the blood water population [3S2X], are given in (22).  In these 2SX equations: pi = 1 – po = [(ρV) + fM]/[1 – fM], pi = 1 – po = [(ρV) + fM]/[1 – fM], koi = [pi/(1 - pi)]kio, κDio  q2Di – Do, and arg(κDio)  q2(Di – Do).  The quantities (ρV), κDio, and arg(κDio) are, respectively, the apparent ρV product [vi], the diffusigraphic shutter-speed (17), and its’ argument.  The volume fraction of intracellular macromolecules, fM (2,9,22), is usually fixed at 0.2 (9): it is assumed extracellular macromolecules do not significantly displace tissue aqueous solutes.  We immediately see the KA model – like all s-s approaches - cannot separate the ρV product.  This is a significant shortcoming.

As originally conceived, the KA model postulated constant single-valued, intrinsic diffusion coefficients for intra- and extracellular water molecules [Di and Do: Din and Dout in Fig. 3] in the absence of exchange: i.e., these would be time-independent.  However, the Fig. 3b simulations suggest this is not the case.  Until the well-mixed water condition is achieved [~20 ms in Fig. 3b], Do, and particularly Di, vary considerably.  Almost instantly, Do limits at some value determined mostly by ρ and V, and Di eventually limits at zero.  If there were closed-form non-empirical expressions describing these time-dependences, they could be incorporated into Eqs. [S16-S20].  But, alas, there are none.  If we could use data from only tD values sufficiently large to render the un-mixed water period insignificant, there would be only three unknowns: kio, Do, and ρV.  But, this is not the case for tD values commonly employed.  As we show in the text, tD(well-mixed) could be even greater than 20 ms (80).

We could take recourse in the shutter-speed laws (2).  Thus, in the vanished s-s condition, Vs-sL [κDio = 0; formerly FXL], D = piDi + poDo [Gaussian diffusion].  However, Vs-sL obtains at only tRW [tD] = 0 and the Di/Do cross-over at tv [~5 ms in Fig. 3b].  For any general case, we can’t know the cross-over D and tv values.  Besides, we have three unknowns, pi, Di, and Do.  As indicated in the text, arg(κDio) changes sign as the system passes through the cross-over Vs-sL.  In some scenarios, this is significant (2).  For the small s-s regime, Ss-sR [κDio < k ( kio + koi); formerly FXR] and large s-s regime, Ls-sR [κDio > k; formerly SXR] conditions, we must employ the complete model Eqs. [S16-S20].  But, the last paragraph shows this is not possible.

The large s-s limit, Ls-sL [κDio >> k; formerly SXL] condition, however, is qualitatively different.  This is particularly manifest in the SDE-CG decay (17).

We note the b axis has different significance for the SDE-CG and [more common] -CT acquisitions.  For CT, b represents q2 while for CG, it represents tD, Eq. [S16].  In the CT decay, the trans-cytolemmal water exchange condition continually changes: varying exchange condition [VXC].  However, in the CG plot, the exchange condition is invariant for the entire decay: constant exchange condition [CXC].  The only way to change the condition is to change the wave number: q2 plays the CA role.  For CA s-s studies, VXC plot analogs are shown in (2,5,10), and CXC plot analogs in (4,7).

In the initial period, the CT and CG decays are identical (8,17).  At some b value, however, they diverge, and the CG decay becomes articulated: it appears more bi-exponential (8,16,17,48,49).  [By definition, the Ls-sR condition obtains when the temporal decay [CG] is detectably bi-exponential (2).]

When the system is actually in the Ls-sL condition, q2Do = q2Do + koi, and q2Di = q2Di + kio (2).  We have no analytical expression for Do.  But, we have seen once tD > tWM, Di limits at zero [Fig. 3b].  Thus, q2Di = kio.  If tD is sufficient that all 1H2Oo coherence has de-phased, the decay is linear [in the semilog plot], and kio is directly determined from the negative of the slope.  The decay has gone through a non-linear/linear [non-Gaussian/pseudo-Gaussian] switchover.  The V product value is not obtained, let alone separated.

This approach has been used in a CG study of gel-suspended isolated glioma cells (16).  As is the case for such samples, the situation is particularly favorable, pi  0.05 (16) and thus koi is very small.  This method has been attempted for anesthetized rat brain in vivo [discussed in (8)].  However, it is not clear that the Ls-sL condition can be achieved for in vivo human brain CG acquisitions.

Another popular related DWI method for mapping trans-membrane water exchange is filter exchange imaging [FEXI] (89-92).  However, FEXI measures only the apparent exchange rate [AXR], which is not a rate, and which confounds kio with , V, and fM (1).  This is because FEXI is a form of what physical chemists term a “chemical relaxation” approach.  As such, it can determine only k [ kio + koi, in the present context].  By itself, it cannot separate kio from koi: V and  must be determined independently of kio to access xcMRNKA, Eq. [1].  Furthermore, FEXI analysis almost always assumes two constant, single-valued D quantities, which must differ considerably: FEXI analyses usually assume Do >> Di.  The invalidity of these assumptions was shown in this work.  In fact, at tD < tv, Do < Di [Figs. 3b and 3e].  FEXI gives unrealistically small kio values for brain gray and white matter (8).  This may be because the crucial FEXI chemical relaxation period often begins near tv, when Di and Do have identical magnitudes (Figs. 3b,3e).

S.VIII.  References found in only SM.

- Tanemura M. Statistical distributions of Poisson Voronoi cells in two and three dimensions.  FORMA  2003;18:221-247.

- Barber CB, Dobkin DP, Huhdanpaa HT.  The Quickhull algorithm for convex hulls.  ACM Trans. Math. Soft.  1996;22:469-483.

- Olesen JL, Østergaard L, Shemesh N, Jespersen SN.  Diffusion time dependence, power law scaling, and exchange in gray matter.  NeuroImage  2022;251:118976.

- Lampinen B, Szczepankeiwicz F, van Westen D, Englund E, Sundgen PC, Latt J, Stahlberg F, Nilsson M.  Optimun experimental design for filter exchange imaging: Apparent exchange rate measurements in the healthy brain and in intercranial tumors.  Magn. Reson. Med.  2017;77:1104-1114.

- Åslund I, Nowacka A, Nilsson M, Topgaard D.  Filter-exchange PGSE NMR determination of cell membrane permeability.  J. Magn. Reson.  2009;200:291-295.

- Bai R, Li Z, Sun C, Hsu Y-C, Liang H, Basser PJ.  Feasibility of filter-exchange imaging (FEXI) in measuring different exchange processes in human brain.  NeuroImage  2020;219:117039.

- Khateri M, Reisert M, Sierra A, Tohka J, Kiselev VG.  What does FEXI measure?  arXiv  2022;2202,00496v1.  [ doi.org/10.48550/arXiv.2202.00496 ]