Metabolic Activity Diffusion Imaging [MADI]:

II.  Non-Invasive, High-Resolution Human Brain Mapping

of Sodium Pump Flux and Cell Metrics

Charles S. Springer, Jr.,*a-e Eric M. Baker,a† Xin Li,a,b† Brendan Moloney,a† Martin M. Pike,a,d† Gregory J. Wilson,f† Valerie C. Anderson,a Manoj K. Sammi,a Mark G. Garzotto,g,h Ryan P. Kopp,g,h Fergus V. Coakley,i William D. Rooney,a,d,j Jeffrey H. Makik

†These authors, listed in alphabetical order, contributed equally.

aOregon Health & Science University [OHSU] Advanced Imaging Research Center, bOHSU Brenden-Colson Center for Pancreatic Care, cOHSU Knight Cancer Institute, dOHSU Department of Biomedical Engineering, eOHSU Department of Chemical Physiology and Biochemistry, fUniversity of Washington Department of Radiology [current: Bayer Healthcare, Radiology], gPortland VA Center, Department of Urology, hOHSU Department of Urology, iOHSU Department of Radiology,  jOHSU Department of Behavioral Neuroscience, kUniversity of Colorado Anschutz Medical Center Department of Radiology.

*Corresponding author:

Charles S. Springer, Jr.

Advanced Imaging Research Center, L452

Oregon Health and Science University

3181 S.W. Sam Jackson Park Road

Portland, Oregon 97239

503-418-1531

springer@ohsu.edu

Supplementary Material

Supplementary Material [citations 1-88 in main text]:

S.I.  Parameter Extraction.

The molecular displacement and SDE decay library curves are numerically generated.  There is no single equation available to calculate them.  They each have three variable parameters, kio, , and V.  Figure 3 suggests library SDE decays can be sufficient to describe experimental SDE-CT data from essentially any biological tissue.  The forward problem seems in hand.  Solving the inverse problem – extracting the best parameters for any voxel experimental SDE decay – is difficult.  Given the relatively smooth [featureless] decay curves, it is possible very similar curves may have quite different parameter sets.  We are most likely facing a clustered parameter space.  The discrete libraries may mitigate the “ill-conditioned” inverse problem.  A “brute force” [i.e., non-iterative] approach – direct least squares comparison of each voxel decay with all library entries in a grid search (36) – avoids the specific pitfalls of local error surface (89) minima to which gradient descent solvers are prone.  The global minimum can still be broad [reflected in Fig. 5c and Table 2].  This method was used for the human brain MADI mapping in Figures 4 and 6, and is now discussed in detail in the Results section.

Though there are analytical [i.e., equation-based] methods for fitting experimental SDE decays, these are all empirical: the parameters cannot be assigned as actual physical properties that might be independently assayed.  Furthermore, one or two parameter expressions are insufficient.  Gaussian diffusion can be characterized with one parameter, the apparent diffusion coefficient [ADC], in the equation: ln[S(b)/S0] = e-b(ADC).  However, Gaussian behavior does not obtain for almost any tissue [Fig. 3].  The diffusion kurtosis expressions, Eqs. [2] and [3], have two parameters; Dap and Kap.  However, as we indicate in the text, these also are insufficient for many tissue decays.

There have been many demonstrations that three parameter empirical expressions are sufficient to describe experimental SDE decays (87).  Perhaps the most popular is the general bi-exponential Equation [S1], where pA is the mole fraction [“population”] of molecules exhibiting diffusion coefficient DA,

SbS0 =  pAe-bDA+ 1- pAe-bDB 				[S1]

and all other molecules exhibit DB.  This expression has three variable parameters: pA, DA, and DB, which are assumed constant [pB = 1 – pA].  It implicitly assumes there is no molecular exchange between the populations [i.e., kAB = 0], which is almost never the case.  More importantly however, every realistic tissue diffusion goes through an essentially infinite number of D values (13).  The significant hazards of inappropriate empirical bi-exponential interpretations are quite perilous (90).  Attempts to assign physical quantities to pA, DA, and DB can lead to very serious errors (8,80,90).  These are empirical parameters.

Every SDE-CT library entry is labeled with its set of three physical parameters: kio, , and V.  However, each of these decays could be fitted very well with Eq. [S1] and thus also labeled with its set of three empirical parameters: pA, DA, and DB.  Voxel experimental decays could be also fitted with Eq. [S1].  The experimental pADA,DB parameter set could be matched with the best library pA,DA,DB set, using the library as essentially a look-up table.  Some weighted sum of the three empirical parameter differences could be used to determine the best match.  The latter could then be assigned with its absolute, fundamental parameters: kio, , and V.  In this way, kio, , and V parametric aMRI maps could be produced.  Such an approach does not seem to us to offer any significant improvement.

The Kärger/Andrasko [KA] model modifies Eq. [S1], with only one additional parameter, to allow steady-state exchange of molecules between populations A and B.  This creates in effect an effectively infinite number of exponential terms.  This model and its descendants [notably FEXI; filter exchange imaging] are elaborated in the companion paper [(13) S.VII.].

S.II.  The Outer Edges of b-Space.

As we indicate in the Discussion section, DWI signals at large b values could be very useful for the inverse problem of extracting aMRI parameters from experimental data.  Exploration of ultra-high b-space has been relatively rare in the in vivo DWI literature.  However, there has been some.  Figure S1a shows three early brain GM studies.  Their semi-log b-space SDE decays are re-plotted, and superimposed, in panel a.  The red diamonds are from a 1999 rat 63 μL ROI study reaching bmax = 64,000 s/(mm)2 (62).  Important details are listed in Table. S1.  The ROI is indicated with the white outline in the upper inset coronal-equivalent image.  The green diamonds are from a 2003 rat 125 μL ROI study reaching bmax = 580,000 s/(mm)2 (60).  The ROI is indicated with the white outline in the lower inset image.  To achieve this [as far as we are aware] record bmax, the investigators employed a SDE-CG acquisition - with a maximum tD of 1177 ms [the 1999 study used the more common SDE-CT approach].  The green diamond decay resembles the Kärger/Andrasko [KA] model large-shutter-speed regime [Ls-sR] condition (8,13,80,91).  The black diamonds are from a 2000 human cerebral cortex 3300 μL ROI study reaching bmax = 3,800 s/(mm)2 (92).  It is remarkable how well the decays from these three disparate studies are superimposed – so much so, the black points are hard to discern [we zoom below].  This demonstrates the expected equivalence of SDE-CT and -CG acquisitions at smaller b values (13).  The smaller bSDE-value [q2tD] is a rather well-normalized.  This is very important, and could be of great use to us.  The disparate B0, Gmax, , and q2max values for these acquisitions are given in Table S1.  It is also important to note, the two different rat brain decays

Figure S1.  Brain tissue semi-log SDE DWI decays to ultra-large b values; (a) 600,000 s/(mm)2, (b) 70,000 s/(mm)2.  In panel a, the black diamonds are from an ROI in awake, human cerebral cortex (92), the red (62) and green (60) diamonds are from  anesthetized rat brain ROIs, which are shown in white frames in the inset images; upper and lower, respectively.  The decays are essentially perfectly superimposed, even though the black and red data were acquired with SDE gradient [-CT], while the green data with diffusion time [-CG]-incrementation.  The black and red panel a data are zoomed in panel b as dark gray and red circles, respectively.  In addition, the white circles are data from a WM ROI in the same human brain (92), while the light gray circles represent SDE-CT data from the left cingulate gyrus of a different awake, resting human brain [red ROI in inset sagittal image] (74).  There is meaningful signal [S] at very large b values.  For pertinent acquisition parameters, see Table S1.

[red and green points] exhibited essentially identical diffusivity decreases immediately upon sudden death induced in the scanner.  Details on this are also found in the companion paper (13).

Panel b zooms two of the panel a decays, and adds two others.  The black and red diamonds of panel a are changed to dark gray and red circles, respectively.  That is, the dark gray circles arise from the 3300 μL ROI in awake, resting human cerebral cortex GM (92), while the red circles arise from the 63 μL ROI in anesthetized rat brain that is mostly GM (62).  Even in this zoomed plot, they are still essentially perfectly superimposed.  [Our library matched those human cGM data well (13).]  The white circles arise from a 3400 μL white matter [WM] ROI of the same human brain (92).  This illustrates a typical difference for brain cGM and WM decays.  The latter have smaller diffusivity.  The light gray circles in Fig. A10b arise from a 220 μL ROI in the awake, resting human left cingulate gyrus.  This is from a more recent SDE-CT study; most likely the record human brain bmax, almost 40,000 s/(mm)2 [Table S1], using a specially designed connectome human brain PFG coil array (74).  This ROI [red in the inset sagittal image slice] most likely contains both GM and WM, and the decay splits those of the dark gray and white circles.

These studies are very important.  They show there is still meaningful brain signal [3 - 20% of S0] at b = 10,000 s/(mm)2.  Moreover, Fig. 3 suggests most biological tissues have diffusivities smaller than that of brain GM, and thus even greater signal at any given b value.  There would be, of course, even more signal at 7,000 s/(mm)2.  Together, the Fig. S1 results further suggest such a b value could be reached with either SDE-CT or -CG [green diamonds, panel a] acquisitions.  For clinical scanners, 7,000 s/(mm)2 may be achievable with SDE-CT.  Larger b values may require SDE-CG – or a hybrid CG/CT method.  For the SMS 3T instrument we employed for the SDE-CT brain aMRI maps [Fig. 4], b = 7,000 s/(mm)2 requires tDmax of only 44 ms [compared with the ~36 ms we used].  Our current drms time-course library [tRW,max = 80 ms] (13) can facilitate SDE-CT decay simulations out to bmax = 8000 s/(mm)2 [= q2maxtD = (105 (mm)-2)(8x10-2 (s))].  And, our library SDE decays are simulation artifact-free down to S ≤ 0.015 S0 [1.5%].  Matching high bmax data could produce more precise kio, , and V maps.

The Fig. S1a green SDE-CG decay merits further discussion.  The authors (60) asserted this plot reveals the trans-cytolemmal water exchange system to be in the KA model Ls-sL [formerly, SXL] condition (8,13).  That is: [q2D =] Dio >> k [= kio + koi].  The q2 value 0.49 (μm)-2 [Table S1] is sufficient to achieve this.  This KA condition is qualitatively different from all others, which cannot be analytically solved (13).  In an SDE-CG decay, the abscissa essentially represents tD [b  tD].  When tD becomes sufficient that the 1H2Oo coherence has dissipated, the decay slope becomes –kio (13).  The decay is no longer diffusion-limited, but efflux-limited.  This is a natural decay articulation; a non-Gaussian/pseudo-Gaussian switchover [a non-linear/linear switchover in the semi-log plot].  The cellular efflux rate constant can then be determined directly from the now-linear decay.  From the green decay limiting slope, Leibfritz and co-workers determined kio = 1.61 (0.08) s-1 for anesthetized rat brain GM [n = 3] (60).  This is the most precise early brain kio measurement, and is in excellent agreement with other [less precise] GM model determinations [Table 3].  However, all these literature reports still used 2SX shutter-speed analyses.  Difficulties with these are discussed in (13) S.VII.

When Gruetter and co-workers made the same assumption for their Fig. S1a red SDE-CT decay, they obtained kio = 8.55 (0.30) s-1 for anesthetized rat brain GM (62).  As shown in Table 3, this disagrees with the other GM model measurements.  This is likely because of either: a.) their acquisition is SDE-CT, or b.) their bmax is only 70,000 s/(mm)2 [Table S1].  If it is the latter, this means the SDE-CG switchover occurs at b > 70,000 s/(mm)2 for anesthetized rat brain GM.  If we could achieve the Ls-sL condition [i.e., reach the switchover] with a clinical instrument, we could apply SDE-CG diffusion encoding to the entries in our molecular displacement [drms] library (13).

S.III.  References found only in SM.

- Li X, Cai Y, Moloney B, Chen Y, Huang W, Woods M, Coakley FV, Rooney WD, Garzotto MG, Springer CS.  Relative sensitivities of DCE-MRI pharmacokinetic parameters to arterial input function (AIF) scaling.  J. Magn. Reson.  2016;269:104-112.

- Mulkern RV, Balasubramanian M, Maier SE.  On the perils of multiexponential fitting of diffusion MR data.  J. Magn. Reson. Imag.  2017;45:1545-1547.

- Hills BP, Godward J, Manning CE, Biechlin JL, Wright KM.  Microstructural characterization of starch systems by NMR relaxation and Q-space microscopy,”  Magn. Reson. Imag.  1998;16:557-564.

- Clark CA, Le Bihan D.  Water diffusion compartmentation and anisotropy at high b values in the human brain. Magn. Reson. Med.  2000;44:852-859.

- Huang SY, Tobyne SM, Nummenmaa A, Witzel T, Wald LL, McNab JA, Klawiter EC.  Characterization of axonal disease in patients with multiple sclerosis using high-gradient-diffusion MR imaging.  Radiology  2016;280:244-251.

S.IV.  Acronyms and Symbols.

A	cell surface area

ADC	apparent diffusion coefficient

ATP 	adenosine triphosphate

A/V	cell area/volume [claustrophobia] ratio

AWC active trans-membrane water cycling

AXR	 apparent exchange rate

ae	membrane “concentration” parameter

aMRI activity MRI

argκXio  s-s argument

αi	contracted Voronoi cell i annulus width

*	global  target

αi*	target i

i(lim)	i upper limit

perpendicular distance to gap center

BBB	blood-brain-barrier

B0	main magnetic field

b	acquisition diffusion-encoding potential

CA	contrast agent

CAo	extracellular CA

[CAo] CAo concentration

CG	constant G, tD incrementation

CI	confidence interval

CT	constant tD, G incrementation

CXC	 constant exchange condition

Ci	standard Voronoi cell i

Ci(cell)inner contracted Voronoi cell i

Ciannannulus of Ci(cell)

ci	seed point of Ci

γ	proton gyromagnetic ratio

D	diffusion coefficient [DT magnitude]

DA 	intrinsic water D in compartment A

DA	apparent DA

Dap	apparent D from Kurtosis Equation

DB	intrinsic water D in compartment B

DB	apparent DB

D 	diffusion tensor

DCE	dynamic-contrast-enhanced

DTI	diffusion tensor imaging

DWI	diffusion-weighted imaging

D-w	diffusion-weighted

Din	intracellular water D [no exchange]

Din	apparent Din

Dn	nanoscopic D

Dout	extracellular water D [no exchange]

Dout	apparent Dout

DWM	Din or Dout limit at large tRW

D0	pure water diffusion coefficient [37oC]

D	absolute Din, Dout difference

d	diffusional water molecule displacement

drms	root-mean-square of d

ECV	tissue extracellular volume fraction [ve]

δ	PFG duration

diffusion interval

ap	apparent

E	expectation

FDG	2-deoxy-2-18Fluoro-D-glucose

FEXI	filter exchange imaging

FOV	field-of-view

FWHM  full width at half maximum

FXL	fast-exchange-limit

fL  [(μm)3]

fMRI	functional MRI

fM	intracellular macromolecular v

Φ	subset of seeds

spin precessional phase

G	magnetic field gradient tensor

G	G magnitude

GLUT  glucose transporter

GM	cerebral gray matter

gamma distribution

HPC	 high power computing

H2Ofree pure water

H2Oin  intracellular water

H2Oout extracellular water

[H2Oi] H2Oin concentration, 0.042 nmol/pL

1H2O	 water proton MR signal

IF	isotropic fractionation

IVIM	 intra-vascular incoherent motion

KA	Kärger/Andrasko

Kap	apparent Kurtosis coefficient

k	steady-state water exchange rate constant

kAB	unidirectional water rate constant, AB

kio	cellular water efflux k (1/i)

kio(a)  active kio contribution

kio(p)  passive kio contribution

koi	cellular water influx k

kpe	capillary CA efflux k

kpo	capillary water efflux k

Ko+	extracellular potassium ion

K-Ras  Kirsten rat sarcoma virus oncogene

fraction of

кDio	diffusigraphic s-s

кFio	flow s-s

κXio	[X = D, F, 1]

к1io	longitudinal s-s

Ls-sL  large κXio limit [formerly, SXL] condition

Ls-sR  large κXio regime [formerly, SXR] condition

ls	RW step length

lsrms	root-mean-square ls

Menc	PFG [zeroth] moment

MR	magnetic resonance

MRI	MR imaging

cMRenz cellular metabolic rate for enzyme process = tMR/ρ

tMRenz tissue metabolic rate for enzyme process = ρcMR

m	number of membranes crossed in RW step

N	number of samples

NA	normal-appearing

NKA	Na+,K+-ATPase (sodium pump)

NXL	no-exchange-limit

n	number of seed points

oxphos  oxidative phosphorylation

domain of Voronoi cells

pop	populated [encompassing]

sim	simulation

src	source

spin precessional [angular] frequency

PET	positron emission tomography

PFG	pulsed G

PVA partial volume averaging

P(n)	probability of finding n seed points in

p	tissue mole fraction (“population”)

p	apparent p

pA	apparent p of water in compartment A

pB	apparent p of water in compartment B

pp	water molecule permeation probability

pL  [1000 (μm)3]

PW	membrane water permeability coefficient

PW†	capillary water permeability coefficient

PW(a)  active cytolemmal PW contribution

PW(p)  passive cytolemmal PW contribution

q	PFG wave number [spatial frequency]

RF	radio frequency

ROI	region-of-interest

RW	random walk

R1	longitudinal relaxation rate constant

R1i	intrinsic 1H2Oi R1

R1o	intrinsic 1H2Oo R1

R1o0	R1o in the absence of CAo

r	water molecule position after RW

r0	starting water molecule position

r1	longitudinal relaxivity

r1o	CAo r1

cell (number) density

(V)	 apparent V product

S	tissue 1H2O transverse signal strength

S(b)	S at b

S0	Boltzmann S

S†	capillary surface area per unit tissue

SD	standard deviation []

SDE	 single diffusion encoding

SE	standard error [/(N)]

SGLT  sodium/glucose co-transporter

SMS	 Siemens Medical Systems

S/N	signal to noise ratio

Ss-sR  small κXio regime [formerly, FXR] condition

SXL	slow-exchange-limit

SXR	slow-exchange-regime

s-s	trans-cytolemmal MR shutter-speed (кXio)

T	Tesla

TE 	pulse sequence magnetization echo time

TI	pulse sequence inversion time

TR	pulse sequence repetition time

Tr[D]	  trace of D

TT	total acquisition time

T1	longitudinal relaxation time constant

T1-w	T1-weighted

T2	transverse relaxation time constant

tD	diffusion time

tRW	RW duration

tV	tD when κDio vanishes

tWM   tD when the “well-mixed” condition is met

ts	RW step time

i	mean H2Oi molecule lifetime (1/kio)

V	cell volume

V()	 volume

Vs-s  vanished s-s (formerly, FXL) condition

VXC	 varying exchange condition

v	tissue volume fraction

vb	blood volume fraction

ve	ECV

vi	intracellular v [V]

WM	cerebral white matter

x	water cycling stoichiometry [H2O/ATP]

x	x for individual water co-transporter

x	position of random walking molecule

xk	x after RW step k

[Xi]	intracellular X concentration

[Xo]	extracellular X concentration

[Xt]	tissue X concentration

X	ROI- or voxel-average of X

2SX	two-site-exchange

3S2X  three-site-two-exchange

| Table S1. Large b-Value Brain Literature Studies |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| brain | B0 [T] | ROI [μL] | bmax [s/(mm)2] | SDE | Gmax [mT/m] | [ms] | tDmax [ms] | q2max [(μm)-2] | TT [min] | Ref. |
| rat | 9.4 | 63 | 70,000 | CT | 300 | 14 | 110 | 0.38 |  | (62) |
| rat | 4.7 | 125 | 580,000 | CG | 151 | 10 | 1177 | 0.49 |  | (60) |
| human | 1.5 | 3300 | 3,800 | CT | 40 | 151 | 2.7 | 0.07 | 9 | (92) |
| human | 3.0 | 220 | 37,000* | CT | 293 | 8.7 | 117 | 0.01* |  | (74) |
| *These values are erroneously under-reported in (74). They are herein corrected – in consultation with the authors. For the correct qmax, see (93). |  |  |  |  |  |  |  |  |  |  |